# Types of maintenance

Maintenance refers to actions taken to:

- preserve the optimal condition of IT infrastructure

- ensure that agreements with external suppliers are in place to support the IT infrastructure, should problems occur

- upgrade the existing IT infrastructure to minimise future risks to business continuity

- fix problems as they occur.

Maintenance concentrates on the critical hardware and software of the organisation. However, it must also be considered in relation to non-critical hardware and software.

## Hardware maintenance

### Preventative and reactive maintenance

Preventative maintenance refers to actions taken to maintain hardware on a periodic basis, to prevent problems occurring in the future that may interrupt business continuity. An example of preventative maintenance is cleaning a workstation hard drive. We will look at preventative maintenance in more detail later in this topic. Upgrading elements of infrastructure is also a part of preventative maintenance.

Reactive maintenance refers to actions taken to fix hardware problems after the problems have occurred. An example of reactive maintenance is replacing a cable after it has proved to be defective.

## Software maintenance

There are different types of software maintenance. For example, an organisation may have custom-built software, which needs to be maintained by its programmers. Software provided by external suppliers may require such things as patches, version updates and driver updates to be provided and installed. Some examples of different types of software maintenance follow.

Preventative maintenance may involve modifications to detect and correct code that may cause errors in the future. These types of changes do not affect normal software functions, but they do affect the way that the code works behind the scenes. An example of this type of maintenance is adding new code to further validate input data.

Reactive maintenance refers to fixing software bugs after they have produced inaccurate results, or have caused processing to stop.

### Adaptive and perfective maintenance (or refinement)

Adaptive maintenance refers to upgrades to the software in response to changes in business requirements. An example of this is altering software to make it run on an Intranet.

Perfective maintenance or software refinement refers to changes to software that result in better performance or easier maintainability. An example of this is a change in software to provide better usability.

QUESTIONS

Additional Resources:

Click BBC

http://www.google.com.au/search?hl=en&source=hp&q=%E2%80%9CClick%E2%80%9D+from+BBC&aq=f&aqi=&aql=&oq=&gs_rfai=

Twitter

http://www.google.com.au/search?hl=en&safe=active&q=twitter&aq=f&aqi=g10&aql=&oq=&gs_rfai=

Push email

http://www.google.com.au/search?hl=en&safe=active&q=push+email&aq=f&aqi=g10&aql=&oq=&gs_rfai=

Instruction Manual

http://www.google.com.au/search?hl=en&safe=active&q=Instruction+Manual+template&aq=f&aqi=g2g-m1&aql=&oq=&gs_rfai=

- Give examples of maintenance on non-critical hardware?

- Cleaning dust and dirt from the keyboard

- Clean ball of the mouse, clean bottom of the mouse

- Defragment the computer

- Provide instructions on maintenance of:-

  - Computer screen:

- Check the monitor’s manual before you start cleaning to see if the screen has any type of protective cover.

- Turn the monitor off before starting

- Use a soft cloth and household window cleaning solution, or a LCD/Plasma cleaner to clean the monitor screen

- Pour or spray the solution on the cloth (not the screen)

- Use the cloth to wipe away dirt from the monitor

  - Trackball mouse:

- Turn off the computer

- Remove the mouse from the computer

- Remove the trackball

- Unlock the disk that’s holding the trackball in place, by either rotating or slide in one direction

- Clean the ball and dust inside the mouse

  - Keyboard:

- Turn off computer

- Remove keyboard from the computer

- Clean keycaps with cotton swabs and a cleaning solution

- Use cleaning solution on cloth and wipe down keyboard

  - Workstation hard drive:

- Periodically run disk clean up

- Keep workstation environment and hard drive cool and well ventilated

- Periodically run disk defragmentation

- Diagnostic utilities

  - Advise the recommended termination for an Ethernet cable:

T568A and T568B termination

These assignments are named T568A and T568B and they define the pin out, or order of connections, for wires in 8P8C (often incorrectly referred to as RJ45) eight-pin modular connector plugs and sockets. Although these definitions consume only one of the 468 pages in the standards documents, a disproportionate amount of attention is paid to them. This is because cables that are terminated with differing standards on each end will not function normally.

TIA/EIA-568-B specifies that horizontal cables should be terminated using the T568A pin/pair assignments, "or, optionally, per [T568B] if necessary to accommodate certain 8-pin cabling systems." The primary colour of pair one is blue, pair two is orange, pair three is green and pair four is brown. Each pair consists of one conductor of solid color, and a second conductor which is white with a stripe of the same color. The specific assignment of pairs to connector pins varies between the T568A and T568B standards. The primary thing one has to be careful of is not to accidentally wire the ends of the same cable according to different configurations (unless one intends to create an Ethernet crossover cable).

  - Why is there A and B standards?

There is an A and B standard when it comes to using Ethernet and phone lines. There is also the fact that as time has changed technology and the way things get done changes, so a need has developed to have more than one standard, with different areas and places already having existing connection types.

- Give examples of preventative maintenance on Windows 7 OS:

Disk clean and disk defragment

- Give examples of reactive maintenance on Windows 7 OS:

System restoration, removal of unnecessary software and reinstall or reboot Windows 7

- Give examples of adaptive maintenance on Windows 7 OS:

Windows 7 updates, upgrade Windows 7 service pack

- Give examples of perfective maintenance on Windows 7 OS:

Patching software and registry

- Create an instruction manual for installing a software application that allows you to save and later replay an internet radio station program on IT e.g. “Click” from BBC, include screen captures

Free And Easy: Freecorder Records Streaming Audio
Freecorder works great. For those who have basic needs for recording streaming audio, such as your favourite online Internet radio station, this application can easily do the job. Instructions:

- Click on the download button

- Choose to run setup.exe file

- Choose the destination to save the software

- Accept any terms of the software, if any

- Follow the installation wizard until finished

- When you install Freecorder for the first time, you are automatically taken to a configuration screen which allows you to easily choose:

- The folder you want your recorded files

- Whether you want to be prompted for a file name before or after recording

- The output quality (64Kbps or 128 Kbps)

- An option to eliminate silence from streams you are recording

- An option to optimize mixer volume for recording

- The option to choose which sound card to record from (if you have more than one)

- The option to decide the source of your audio (line in, Auxiliary, CD or microphone)

- Twitter

  - Explain “twittering”

Twitter is a social networking and micro blogging service, owned and operated by Twitter Inc. that enables its users to send and read other users' messages called tweets. Tweets are text-based posts of up to 140 characters displayed on the author's profile page. Tweets are publicly visible by default; however senders can restrict message delivery to their friends list.

  - Create an explanation of the instructions for creating a twitter for a corporate marketing client

- Go to The Twitter homepage and scroll all the way to the bottom. Click on the "Get Started" button.

- Create a username and password. Twitter will automatically check the availability of your username. Once you find one that is unique, enter your email address and continue.

- After you create your account, you can manually see if any of your e-mail contacts are on Twitter. Just enter your e-mail username and password, and twitter will automatically import anyone it recognizes.

- Invite friends. Any emails Twitter didn't recognize, it will display on the next page. You will be able to check different email addresses to send invites to.

- Search for friends. Chances are good that your friends have multiple email addresses, so it is a good idea to enter their name in the search bar and double check.

- Set up your phone. The big advantage of Twitter is that you can update via text messages from your phone. Go to the "Devices" tab on your profile page and then enter your cell phone number. A confirmation message will be sent to your phone, and then you'll be able to make your first update.

http://www.ehow.com/how_4541204_create-twitter-account.html

- Push Email

  - What is “push” email?

Push e-mail is used to describe e-mail systems that provide an always-on capability, in which new e-mail is actively transferred (pushed) as it arrives by the mail delivery agent (MDA) (commonly called mail server) to the mail user agent (MUA), also called the e-mail client. E-mail clients include smart phones and, less strictly, IMAP personal computer mail applications.

  - Explain to a client how to get push email happening on a recommended device (show diagrams).

http://lifehacker.com/398526/set-up-push-email-contacts-and-calendar-on-your-iphone-for-free

http://apcmag.com/set_up_push_email_on_the_iphone_with_any_email_address.htm

  - Explain a contingency plan if push email is not working.

- Use your normal email

- Use SMS

- Use normal voice call